Report

Submitted by:

Adwaith A(CB.SC.U4CSE23204)

Adwaith Raveendran(CB.SC.U4CSE23603)

1. Objective

Implement and demonstrate a trained model that classifies plant health from the robot’s camera in Webots, then triggers the appropriate actuator (waterLED / pesticideLED). The emphasis is on a complete pipeline: data → training → deployment → real-time behavior.

2. Simulation Setup

- Robot: Pioneer3-DX with camera (weedCam) at 128×128 (or 256×256)

- Scene: Rows of potted plants; healthy leaves are green, malnourished are white/pale

- Lighting: Neutral daylight (directional + mild ambient), matte brown pots to avoid white/green contamination

3. Dataset Collection

- Temporary controller collect_dataset.py captured center crops from the camera every ~0.4 s while driving past plants.

- Two passes: one facing healthy plants, one facing malnourished plants.

- Saved images to:

- dataset/healthy/*.png

- dataset/malnourished/*.png

- Target count: 50 images per class (worked with as low as 20/class).

4. Features

For each image, we compute a compact vector (10 dims):
1–6. μ/σ of RGB: (μR, μG, μB, σR, σG, σB)
7–8. μS, μV: average HSV saturation & value (computed from RGB)
9. Green-score: mean of max(G−max(R,B)−0.03, 0) — measures “how much greener than others”
10. White-ratio: fraction of pixels with S≤0.30 and V≥0.70 (bright/low-saturation)

Rationale: these encode color intensity, contrast, “greenness,” and “whiteness,” which directly relate to the task.

5. Model & Training

- Model: Logistic Regression (binary), trained with NumPy only; L2 regularization (λ=1e-3).

- Preprocessing: Standardize features using training-set mean/σ.

- Optimization: Gradient descent, lr=0.05, 4000 steps.

- Outputs: plant_color_model.npz containing weights (w,b) and normalization stats (μ,σ).

- Evidence: Training log with decreasing loss and improving accuracy
(Insert screenshot of console showing “step 500 … loss … acc …” → “Saved -> plant_color_model.npz”)

6. Deployment in Controller

- Main controller plant_identifier.py:

  - Loads plant_color_model.npz at startup; prints [model] loaded …

  - State machine: MOVE → ANALYZE → ACT (LED on) → ESCAPE (forward+spin) → MOVE

  - In ANALYZE, grabs a BGRA→RGB center crop (upper foliage band), computes features, predicts:

    - HEALTHY (p<0.6) → waterLED=1

    - MALNOURISHED (p≥0.6) → pesticideLED=1

  - If the model file is missing, uses a heuristic fallback based on the same features (so demo always runs).

(Insert screenshot of Webots console showing [model] → HEALTHY (p=0.73) or [model] → MALNOURISHED (p=0.82).)

7. Results & Discussion

- Real-time inference (≈0 ms at 128×128, pure NumPy).

- Stable decisions in aisle runs; green plants classified healthy, white plants malnourished.

- Advantage: Works without Webots’ recognition boxes; robust to textures because the crop targets foliage and features average over the region.

- Limitation: Color-based—sensitive to extreme lighting and very similar hues. Future work: add data augmentation (brightness/contrast jitter), consider a tiny CNN/ONNX or a classical detector for multiple plants per frame.

8. Reproducibility Steps

- Run collect_dataset.py twice to fill dataset/healthy/ and dataset/malnourished/.

- In a terminal in controllers folder:

- pip install numpy pillow imageio

- python train_color_model.py

- Ensure plant_color_model.npz is placed next to plant_identifier.py.

- Attach plant_identifier.py to the robot and run the world:

  - Console prints [model] loaded …

  - During pauses: [model] → HEALTHY/MALNOURISHED (p=…)

  - LEDs toggle accordingly; robot escapes and proceeds.

9. What I’m Submitting

- Screenshots:

  - Dataset folder structure with images

  - Training console (loss/acc + “Saved -> plant_color_model.npz”)

  - Webots run showing [model] → … (p=…) and LEDs

- Video simulation and the output screenshots of the console

- Code: collect_dataset.py, train_color_model.py, plant_identifier.py

- This report + one slide summarizing features, model, and integration.